Simulation stack:

Python (backend)

Javascript/html (frontend)

Dependencies:

- Fastapi (web server)

- Tensorflow lite (ai inference)

- Pil/pillow (image processing, simulation replacement for opencv)

- Numpy (MAAAATH)

Physics

Kinematics

The robot moves on a flat surface and has a direction it is facing. At any moment, its situation can be described by:

x(t)y(t)θ(t)

where:

- x(t) = horizontal position (cm)

- y(t) = vertical position (cm)

- theta(t) = direction (radians)

- t = time

this is called a state vector.

Motion Model (Unicycle Model)

The robot moves like a vehicle, it can move forward, turn but not slide sideways.

This type of motion is simulated using the unicycle model.

The robot has 2 control inputs:

- v = forward speed (cm/s)

- w = angular speed (rad/s)

The mmotion equations are:

dxdt=v*cosθ

dydt=v* sinθ

dθdt=w

Explanation for the equations above:

Cos(theta) and sin(theta) split the forward movement into x and y directions

If the robot faces to the right, x is the variable changing

When facing up, y is the variable changing.

W controls how fast the robot turns.

Navigation and control

Target direction and angle error

When the robot wants to move to a target point (x,y), it first calculates the desired direction.

The desired heading angle is:

θtarget=tan-1(yt-yxt-x)

Where:

(x,y) = current robot position

(xt, yt) = target position

The heading error is the difference between where the robot should face and where it does face: eθ= θtarget- θ

This value tells the robot:

Positive  turn left

Negative  turn right

Zero  already facing the correct direction

Proportional controller (P-Controller)

To correct the direction smoothly, the robot uses a Proportional controller.

The angular speed is:

w= Kp*eθ

Where:

- w = turning speed

- Kp = proportional constant

- e-theta = heading error

meaning:

- if the error is large  turn faster

- if the error is small  turn slow and gentle

this wasn’t part of our original code, I only realized we need this when making the simulation, so I added this to our actual code as well.

mapping

the robot does not know the size of the panel before mapping. It  determens the boundary by detecting collision in four directions, up, right, down, left

form this, the robot finds:

xmin,xmax,ymin,ymax

These values makes a rectangular area:

A = (xmax – xmin) * (ymax – ymin)

Where A = total panel area

Path planning

The robot plans a path going inwards in a spiral

After each loop of the rectangular area, the boundary shrinks by a fixed amount.

The shrinking distance is defined as:

d=w*(1-o)

Where:

- w = robot width

- o = overlap ratio (0.5 = 50%)

the boundary at loop k are:

xmink= xmin+k*d

xmax(k)= xmax-k*d

ymin(k)= ymin+k*d

ymax(k)= ymax-k*d

And the spiral stops when:

xmin(k)≥xmax(k) or ymin(k)≥ymax(k)

Dirt detection

Each camera image is processed by the neural network.

The network outputs a probability:

P(dirty | image)

If P(dirty) > r then the location is marked as dirty.

R = threshold value

Cleaning route optimization

Distance calculation

The distance between 2 dirty spot I and j is:

dij=(xi-xj)2+(yi-yj)2

Nearest neighbor algorithm

After scanning the solar panel, the robot can have found several dirty spots. Each spot is then positioned as (x1,y1),  (x2,y2), (x3,y3), …. , (xn,yn)

Where n is the total number of dirtyt spots found

The robot then needs to decide in which order to clean them.

Lets say the robot is at position i. we then have to calculate all the distances from the current robot position to the dirty spots found : di1, di2, di3 …

The robot then chooses the smallest distance, which in math is written as:

Next spot=argminj(dij)

This simply means: choose the value of j for which the distance dij is the smallest.

Min  gives the smallest value

Arg min  gives which point produces the smallest value

After choosing an order to go, so 123 etc, the total distance traveled is the sum of all individual distances between consecutive spots.

L= k=1n-1dk,k+1

Which basically means:

L= d1,2+d2,3+…+dn-1,n